# ARBOR: Project Report (Forge)

## 1. Project Introduction

The project is concluded as a single-class instance segmentation by YOLO-seg to automatically detect and segment the crowns of Shihuahuaco (genus Dipteryx) from Peru Amazonian UAV orthomosaics.

## 2. Data Sources and Annotation

The data come from two years.

2023: a set of sites (GSD ≈ 3.65 cm). Total annotated crowns: 1481. 229 Shihuahuaco crowns.

2025: two sites, ISA and CATAHUA (GSD ≈ 3.54 cm). 375 Shihuahuaco crowns.

## 3. Dataset Construction

### 3.1 Tiling by "Ground Metres"

To maintain model perception consistency, we tile the orthomosaics by ground size and then resample to a fixed pixel size, which makes the model GSD-free.

The solution is to tile by ground size and then resample to a fixed pixel size:

- Ground block: 56 m (In some cases, the original 1024-pixel images (at 3.6 cm GSD) do not cover the entire Shihuahuaco crown. Hence, we increase to 56m.）

- Overlap: 50% (stride = 28 m)

- Each block resampled to 1024 px

### 3.2 Samples

Positive samples (complete crowns + tiles): Priority to complete crowns; A tile with crown coverage > 0.5 also counts as a positive.

Negatives samples: 3 × positives; All other crown annotations and random backgrounds.

### 3.3 Train Dataset

- Positives: 2957

- Negatives: 8212

- Total tiles: 11169

## 4. Training Configuration

| Item | Value |
| --- | --- |
| Model | YOLO11m-seg |
| Input size (imgsz) | 1024 |
| Epochs | 150 |
| Patience (early stop) | 30 |
| Number of classes | 1 (shihuahuaco) |

## 5. Evaluation on Held-out Sites

The model is evaluated on two held-out sites excluded from training (16-CONP-MAD-SD-023-15 and GOREMAD-GRRNYGA-DRFFSDFFS-TAHP-MAD-D-008-15). positive = Shihuahuaco (IoU≥ 0.5), Confidence threshold = 0.5.

Total evaluated crowns: 341.

| Metric | Value |
| --- | --- |
| Precision | 0.680 |
| Recall | 0.460 |
| F1 | 0.548 |
| Accuracy | 0.836 |
| TP / FP / FN / TN | 34 / 16 / 40 / 251 |

False positives by species (which look-alikes most often fooled the model):

| Count | Species |
| --- | --- |
| 4 | Copaifera reticulata |
| 3 | Hura crepitans |
| 2 | Parkia nitida |
| 2 | Apuleia leiocarpa |
| 2 | Hymenaea oblongifolia |
| 1 | Brosimum guianense |
| 1 | Pouteria torta |
| 1 | Cariniana estrellensis |

These species can be treated as hard negative samples and utilized in the next stage.

The final deployed model is trained on the full dataset.

## 7. Plugin

Pipeline: sliding-window inference → map masks back to world coordinates → merge adjacent masks and redetect → write out GPKG layer.

Parameters :

- --det-conf 0.25 (low): the detection threshold is deliberately low so that redundant detections from overlapping tiles are all kept, letting the merge step stitch them together without leaving isolated fragments.

- --keep-conf 0.5 (high): after merging, each whole crown is filtered by its highest-scoring member, removing low-scoring whole blobs.

Shape filtering (removes "spiky / thin-strip" spurious detections): --min-area 30 (minimum area in m²), --max-aspect 4 (long/short ratio)

Fragment Re-detection to eliminate incomplete fragments; any merged crown is re-tiled around its centre point for a fresh detection.

QGIS plugin deployment — minimal dependencies: only numpy + onnxruntime + osgeo.gdal + the QGIS API.